- Receive frames:

- If untagged, need to tag VID of frame = PVID

- If tagged already, no need to do anything

- Check if frame allowed to enter into port

- For access port: if VID of frame == PVID, can enter into port

- For trunk port: if VID of frame == port trunk allowed-pass VLAN XX, can enter into port

- For hybrid port: if VID of frame = tagged or untagged list, can enter into port

- Send out frames:

- Check if frame is allowed to exit out of port

- For access port: if VID of frame == PVID, can exit out of port

- For trunk port: if VID of frame == port trunk allowed-pass VLAN XX, can exit out of port

- For hybrid port: if VID of frame = tagged or untagged list, can exit out of port

- Check need to untagged the frame:

- For access port, if VID of frame == PVID, strip off the tag

- For trunk port: if VID of frame == PVID, strip off the tag. Otherwise, remain tagged.

- For hybrid port: if VID of frame = untagged, strip off the tag. Otherwise, remain tagged.

https://support.huawei.com/enterprise/en/doc/EDOC1100096312/a0f4f7e4/port-trunk-allow-pass-vlan

https://support.huawei.com/enterprise/en/doc/EDOC1100064351/923935bf/port-hybrid-untagged-vlan

Setting PVID of access port

https://support.huawei.com/enterprise/en/doc/EDOC1100277028/6907bedb/default-vlan

https://support.huawei.com/enterprise/en/doc/EDOC1000141870/e2b68f76/example-for-configuring-interface-based-vlan-assignment-access-device-used-as-the-gateway

Setting PVID of trunk port

https://support.huawei.com/enterprise/en/doc/EDOC1100096311/299a5aeb/port-trunk-pvid-vlan

Setting PVID of hybrid port

https://support.huawei.com/enterprise/en/doc/DOC1000128405/1327376/port-hybrid-pvid-vlan